UNIVERSITY OF HUDDERSFIELD – FIRE RISK ASSESSMENT

| A: Fire Detection and Warning Systems |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # Building |  |  | ## Area |  | ## Assessed by |  |  |  |  | ## Date |  |
|  |  |  |  |  |  | ### Yes | No |  | Comments |  |  |
| 1 | Is the level of fire detection appropriate to the fire hazards, give occupants early warning and reduce the likelihood of unwanted activations? |  |  |  |  |  |  |  |  |  |  |
| 2 | Are warning devices sufficient to alert all occupants (e.g. people in remote locations)? |  |  |  |  |  |  |  |  |  |  |
| 3 | Are warning devices appropriate for the activities that are taking place? |  |  |  |  |  |  |  |  |  |  |
| 4 | Do warning systems account for occupants with disabilities? |  |  |  |  |  |  |  |  |  |  |
| 5 | Is the fire alarm clearly distinguishable from any other alarm/warning system? |  |  |  |  |  |  |  |  |  |  |
| 6 | Is there means in place to contact the emergency services? |  |  |  |  |  |  |  |  |  |  |
| 7 | Is the system – including all devices linked to it – subject to testing and maintenance undertaken by competent personnel? |  |  |  |  |  |  |  |  |  |  |
| 8 | Are there protocols in place in the event of detection and/or warning system(s) failure? |  |  |  |  |  |  |  |  |  |  |
| 9 | (Others – please state) |  |  |  |  |  |  |  |  |  |  |
| A: FIRE DETECTION AND WARNING SYSTEMS - GUIDANCE |  |  |  |  |  |  |  |  |  |  |  |
| DETECTION |  |  |  |  |  |  |  |  |  |  |  |
| As a minimum, means of escape must be covered by automatic fire detection and manual call points. In areas where greater fire hazards are present (e.g. use of flammable/highly flammable liquids) or where fire could start and spread undetected (e.g. plant rooms) then detection is required to these individual areas. Detectors should be optical smoke, except for areas where heat detectors (e.g. kitchens) or beam detectors (e.g. roof voids) are more appropriate. |  |  |  |  |  |  |  |  |  |  |  |
| WARNING |  |  |  |  |  |  |  |  |  |  |  |
| Warning devices must be clearly audible in all areas that could potentially be occupied. Particular attention should be paid to store rooms, inner rooms, plant rooms and toilets. Where ambient noise levels may make the hearing of a warning device difficult (e.g. workshop environment) or where individuals have hearing impairment, alternative warning devices must be in place (e.g. visual alarms). The fire alarm signal must be clearly distinguishable from any other warning device (e.g. theft, door alarms) used within the building. |  |  |  |  |  |  |  |  |  |  |  |
| MAINTENANCE AND TESTING |  |  |  |  |  |  |  |  |  |  |  |
| The completion detection and warning system, including any emergency devices linked to it (e.g. magnetic door locks), must be tested weekly for assured function. Each single detection device (detectors and call points) must be tested for activation at least once annually. The complete detection and warning system must also be subject to more thorough test and maintenance as prescribed by BS5839. All inspections and tests must be carried out by competent personnel, familiar with the system and its operation. Records of all inspections and tests must be maintained for at least one year post or, where appropriate, up to the next scheduled test or inspection. |  |  |  |  |  |  |  |  |  |  |  |
| CONTINGENCY |  |  |  |  |  |  |  |  |  |  |  |
| Arrangements must in place for prompt implementation should detection (e.g. temporary devices or regular manual checks, especially overnight) and/or warning (e.g. temporary devices, physical alert) devices fail. |  |  |  |  |  |  |  |  |  |  |  |